CHAPTER I: CRYSTAL STRUCTURE

Content:

- Introduction

- Structure of cubic crystals (SC, BCC, FCC)

- Ligancy and critical radius ratio in ionic crystal

- Miller indices, planes and direction

- Crystal structure Analysis

- Imperfections: point, line, surface & volume (introductory)

- Introduction

  - SHORT AND LONG RANGE ORDER

Order: Presence of some symmetry or correlation in a many-particle system.

Short-range order: It exists till the first or second-nearest neighbors of an atom. It implies non dense, random packing (fig.1).

Figure 1: Short-range order of materials. 		Figure 2: Long-range order of materials.

Long-range order: It exists over long distances. It implies dense, regular packing (fig.2).

Dense, regular-packed structures having long-range tend to have lower energy.

    - Classification Of Matter On The Basis Of Order

Materials can be classified on the basis of order. Gases do not have any order. This is followed by amorphous materials which has only short-range order. Next in line are liquid crystals which have short and long range order in small volumes.

Finally, long range order is observed in crystalline materials.

They are of two types: single crystal and polycrystalline.

.

Figure 3: Classification of materials on the basis of order.

    - Difference Between Single And Poly-Crystals

Single crystal: A single crystal or monocrystalline solid is a material in which the crystal lattice of the entire sample is continuous and unbroken to the edges of the sample, with no grain boundaries (fig.4a).

The properties of a single crystal vary with direction. This is termed as anisotropy. For eg. the modulus of elasticity (E) in BCC iron (fig.4b). The values of modulus of elasticity in BCC iron is different along the edge of the cube and along the body diagonal of the cube giving rise to anisotropy.

(b)

(a)

Figure 4: (a) Pictorial representation of a single crystal. (b) Modulus of elasticity (E) in BCC iron.

Poly-crystal: It is comprised of many small randomly oriented crystals or grains. The properties of a polycrystalline material may/may not vary with direction, depending on degree of texture (orientation). If the grains are randomly oriented, then the properties are same along all directions i.e., isotropic. For eg. the bulk modulus of randomly oriented polycrystalline iron is 210 GPa along all directions (fig.5). However, if all the grains are oriented in a single direction, then the properties will be anisotropic.

Figure 5:

Polycrystalline sample of non-oriented iron.

1.2 LATTICE, BASIS, CRYSTAL STRUCTURE

Space Lattice: A translational periodic arrangement of lattice points. Each lattice point in a lattice has identical neighborhood of other lattice points.

Bravais Lattice: Bravais lattice, studied by Auguste Bravais (1850), is an infinite array of discrete points in three dimensional space generated by a set of discrete translation operations described by:

R = n1a1 + n2a2+n3a3

where ni are any integers and ai are known as the primitive vectors which lie in different directions and span the lattice. For any choice of position vector R, the lattice looks exactly the same.

Motif or Basis: An atom or a group of atoms associated with each lattice point.

Crystal: A translational periodic arrangement of atoms. It is made up of lattice and basis.

Types of crystals

    - Unit Cell and Primitive Cell

Unit Cell      The smallest building block of a crystal, consisting of atoms, ions, or molecules, whose               geometric arrangement defines the symmetry and structure of the entire crystal lattice and whose repetitive translation in space produces the crystal lattice is called the unit cell.

The unit cell is characterized by three lattice constants a, b, c (the lengths of the basis vectors or edges of the unit cell) and by the three angles α, β, γ which separate these vectors from one another. α is the angle between the basis vectors b and c, β between a and c, and γ between a and b (fig. 6).

The lattice constants and the angles between them are called lattice parameters.

The volume of a unit cell is given by

V = a. ( b x c )

Figure 6: Pictorial representation of unit cell.

Primitive Cell

Primitive cell is a unit cell constructed so that it contains only one lattice point. The primitive cell has lattice points only at corners. It does not define the full symmetry of the entire space lattice. There are multiple ways to select a primitive cell (fig.7).

Figure 7: Multiple ways to select a primitive cell.

Non-primitive unit cell: It has lattice Point at corners as well as at other points.

1.3 SYMMETRY

Symmetry: A body is said to be symmetrical when it can be divided into parts that are related to each other in certain ways.

Symmetry Operation: An operation that leads to superimposition of an object on itself.

Symmetry operators: translation, rotation, reflection, inversion.

Symmetry Element: An imaginary geometric entity (line, point, plane) about which a symmetry operation takes place.

Symmetry elements: point, line, plane.

Rotation: An n-fold rotation symmetry means rotation through an angle of 2π/n will repeat the object or motif n times in a full 360⁰ rotation. n =1 means no symmetry. In a crystal, only 2, 3, 4 and 6-folds of rotation are possible.

Reflection: An object with a reflection symmetry will be a mirror image of itself across a plane called mirror plane (m).

Inversion: In this operation, every part of the object is reflected through an inversion center called center of symmetry which is denoted as i. The object is reproduced inverted from its original position.

Combined Operations:

- Screw Axis: A screw axis (helical axis or twist axis) is a line that is simultaneously the axis of rotation and the line along which translation of a body occurs.

- Glide plane: A glide plane (or transflection) is a symmetry operation describing how a reflection in a plane, followed by a translation parallel with that plane, may leave the crystal unchanged.

- Roto-inversion: A rotoinversion axis combines rotation about an axis of rotation with inversion.

1.4 SEVEN DIFFERENT CRYSTAL SYSTEMS AND FOURTEEN DIFFERENT BRAVAIS LATTICES:

Crystals are divided into 7 crystal systems according to their geometric symmetries and into 7 lattice systems according to their Bravais lattices.

A lattice system is generally identified as a set of lattices with the same shape according to the relative lengths of the cell edges (a, b, c) and the angles between them (α, β, γ).

Each lattice is assigned to one of the following classifications (lattice types) based on the positions of the lattice points within the cell: primitive (P), body-centered (I), face-centered (F), base-centered (A, B, or C), and rhombohedral (R).

The 14 unique combinations of lattice systems and lattice types are collectively known as the Bravais lattices.

Thus, solids are classified into 7 crystal systems and 14 Bravais lattices as represented in table 1 and 2.

Table 1: Classification of solids into 7 crystal systems on the basis of geometric symmetries.

Table 2: Classification of solids into 7 crystal systems and 14 Bravais lattices.

- STRUCTURE OF CUBIC CRYSTALS

2.1 CUBIC CRYSTAL STRUCTURES

2.1.1 Parameters of crystal structure:

- Number of atoms per unit cell or Effective number: It is the total number of atoms per unit cell.

- Co-ordination number: It is the number of nearest neighboring atoms to a particular atom in the crystal.

- Atomic radius: It is defined as half of the distance between two nearest neighbors in a crystal of a pure element since any two nearest neighboring atoms touch each other. It is usually expressed in terms of the edge of the unit cell ‘a’.

- Atomic packing factor (Packing density) (APF): Atomic packing factor (APF) or packing density indicates how closely atoms are packed in a unit cell and is given by the ratio of volume of atoms in the unit cell and volume of the unit cell.

2.1.2 Simple Cubic Structure

a

R

Number of atoms per unit cell or Effective number:

There are 8 atoms at the eight corners of the unit cell. Each of these atoms are shared by eight unit cells.

Therefore, effective number of atoms in simple cubic unit cell is equal to

8 (corner atoms) x 1/8 = 1 atom / unit cell.

- Co-ordination number:

Each corner atom is in contact with two other atoms along the edges of the cube.

Hence, the co-ordination number of simple cubic structure is 6.

- Atomic radius:

In a simple cubic structure, the atoms touch each other along the edge of the cube.

Hence, the atomic radius 'R' in terms of the length of the edge of the unit cell 'a' is	a = 2R 												Figure 8: Simple Cubic

Structure

- Atomic Packing Factor

Co-ordinates of atoms in simple cubic structure.

2.1.3 Body-centered Cubic Structure

Figure 9: Body-centered cubic structure

- Number of atoms per unit cell or Effective number:

There are 8 atoms at the eight corners and 1 atom at the center of the unit cell.

Each of the corner atoms are shared by eight unit cells while the center atom entirely belongs to the unit cell.

Hence, the effective number of atoms per unit cell is

8 (corner atoms) x 1/8  + 1 centre atom = 2 atoms / unit cell.

4R

- Co-ordination number:

The body-centered atom is in contact with all the 8 corner atoms.

Hence, co-ordination number of                         body-centered cubic structure is 8.

- Atomic radius: In a body-centered cubic structure, the atoms touch each other along the body diagonal of the cube. Hence, the atomic radius 'R' in terms of the length of the edge of the unit cell 'a' is

3a = 4R  (fig.10).

Figure 10

- Co-ordinates of atoms in Body center cubic structure

- Atomic Packing Factor

Calculation of APF:

2.1.4 Face-centered Cubic Structure (FCC)

- Number of atoms per unit cell or Effective number:

There are 8 atoms at the eight corners and 6 atom at the centre of each face of the unit cell.

Each of the corner atoms are shared by eight unit cells while the face-centered atoms are shared by 2 unit cells.

Hence, the effective number of atoms per unit cell is

8 (corner atoms) x 1/8  + 6 (face-centered atoms) x 1/2 = 4 atoms / unit cell.

- Co-ordination number:

The face centered atom in the front face is in contact with four corner atoms and four other face-centered atoms behind it (two sides, top and bottom) and is also touching four face-centered atoms of the unit cell in front of it.

Hence, co-ordination number of FCC is 12.

Figure 11: Face-centered cubic structure.

- Atomic radius:

In a face-centered cubic structure, the atoms touch each other along the face diagonal of the cube. Hence, the atomic radius 'R' in terms of the length of the edge of the unit cell 'a' is

2a = 4R 	(fig.12).

Figure 12

- Atomic Packing Factor:

Calculation of APF:

Co-ordinates of atoms in FCC.

Fractional Co-ordinates: 000,100, 010, 001, 110,101, 011,111, ½ ½ 0, ½ 0 ½, 0 ½ ½ ,½1 ½ , 1 ½ ½ , ½ ½ 1

Unique Positions: (0,0,0), (1/2,1/2,0), (1/2,0,1/2), (0,1/2,1/2)

IV	  MILLER INDICES, PLANES AND DIRECTIONS

4.1 Miller Indices

Miller indices are used to specify directions nd planes in crystals.

To find the Miller Indices of a plane, follow these steps

- First draw the cube.

- Identify the intercepts on the x- , y- and z- axes.

- Specify the intercepts in fractional co-ordinates. Co-ordinates are converted to fractional co-ordinates by dividing by the respective cell-dimension.

- Take the reciprocals of the fractional intercepts.

- Clear fractions by multiplying or dividing by a common factor.

- Reduce to lowest terms. This final manipulation generates the miller indices (hkl) which (by convention) should then be specified without being separated by any commas or other symbols. The miller indices are also enclosed within standard brackets (….).

- Planes can also have negative intercept e.g. 1, -1/2, 1 Hence, h k l = 1 -2 1. This is denoted as ( 1 2 1 ).

Intercepts :   ½ a , a ,∞

Fractional intercepts :   ½ , 1 , ∞

Miller Indices :   (210)

Intercepts :   ½ a , ∞  ,∞

Fractional intercepts :   ½ , ∞, ∞

Miller Indices :   (200)

Intercepts :   a , a , ∞

Fractional intercepts :   1 , 1 ,∞

Miller Indices :   (110)

Intercepts :   a , a , a

Fractional intercepts :   1 , 1 , 1

Miller Indices :   (111)

Planes having similar indices are equivalent, e.g. faces of the cube (100), (010) and (001). This is termed as a family of planes and denoted as {100} which includes all the (100) combinations including negative indices (fig. 27).

Figure 27: Families of planes {100}, {110} and {111}.

4.1.1 Interplanar Spacing

To find the interplanar spacing for the set of planes with Miller indices (hkl), take the origin on any one plane of this set and erect axes in the directions of crystal axes a, b and c.

The interplanar spacing d for the planes of indices (hkl) is equal to the distance from origin to the nearest plane of the set under question.

The distance d is measured along the normal drawn from origin to the plane (hkl). Let the normal to the plane make angles ,  and   with x-axis, y-axis and z-axis respectively.

The intercept of the plane on the three axes are

OA = a/h, OB = a/k, OC = a/l

where a is the length of the cube edge.

Figure 28: Intercepts made by the plane with miller indices (hkl) along a, b and c axes.

cos   = d/OA = dh/a,

cos   = d/OB = dk/a

cos  =  d/OC = dl/a

For orthogonal axes,

cos2 + cos2 + cos2 = 1

Therefore,

dhkl2h2a2+k2b2+l2c2=1

dhkl=1h2a2+k2b2+l2c2

or

For cubic system, a = b = c. Hence, we get,

dhkl=ah2+ k2 + l2

4.1.2 Crystal Directions

The directions in a crystal are given by specifying the coordinates (u, v, w) of a point on a vector (ruvw) passing through the origin.

ruvw = ua + vb + wc.

It is indicated as [uvw]. For example, the direction [110] lies on a vector r110 whose projection lengths on x and y axes are one unit (in terms of unit vectors a and b).

Directions of a form or family like [110], [101], [011] are written as <110>. Families of directions <100>, <110> and <111> are given in figures 29 and 30.

Figure 29: <100> and <110> family.

Figure 30: <111> family.

To determine a direction of a line in the crystal:

- Find the coordinates of the two ends of the line and subtract the coordinates (Head – Tail) OR draw a line from the origin parallel to the line and find its projection lengths on x, y and z axis in terms of the unit vectors a, b and c.

- Convert fractions, if any, in to integers and reduce to lowest term.

- Enclose in square brackets [uvw].

- The line which passes through uvw will also pass through 2u2v2w and ½ u ½ v ½ w. Hence [uvw], [2u2v2w] and [½ u ½ v ½ w] are same and written as [uvw].

V	CRYSTAL STRUCTURE ANALYSIS

5.1 Bragg Diffraction

- Bragg diffraction was first proposed by William Lawrence Bragg and his father William Henry Bragg in 1913.

- They found that crystals, at certain specific wavelengths and incident angles, produced intense peaks of reflected radiation.

- Both neutron and X-ray wavelengths are comparable with inter-atomic distances (~150 pm) and thus are an excellent probe for this length scale.

- Bragg's law confirmed the existence of real particles at the atomic scale, as well as providing a powerful new tool for studying crystals in the form of X-ray and neutron diffraction.

- W. L. Bragg and W. H. Bragg were awarded the Nobel Prize in physics in 1915 for their work in determining crystal structures beginning with NaCl, ZnS, and diamond.

VI	IMPERFECTIONS: Point, Line, Surface and Volume

6.1 CRYSTAL DEFECTS

A perfect crystal is an idealization; there is no such thing in nature. It is useful to classify crystal lattice defects by their dimension.

- The 0-dimensional defects affect isolated sites in the crystal structure, and are hence called point defects. An example is a solute or impurity atom, which alters the crystal pattern at a single point.

- The 1-dimensional defects are called dislocations. They are lines along which the crystal pattern is broken.

- The 2-dimensional defects are surfaces, such as the external surface and the grain boundaries along which distinct crystallites are joined together.

- The 3-dimensional defects change the crystal pattern over a finite volume. They include precipitates, which are small volumes of different crystal structure, and also include large voids or inclusions of second-phase particles.

6.1.1 Point Defects:

A point defect disturbs the crystal pattern at an isolated site. It is useful to distinguish intrinsic defects, which can appear in a pure material, from extrinsic defects, which are caused by solute or impurity atoms.

Intrinsic defects

An intrinsic defect is formed when an atom is missing from a position that ought to be filled in the crystal, creating a vacancy,

or when an atom occupies an interstitial site where no atom would ordinarily appear, causing an interstitialcy.

The two types of intrinsic point defects are shown in fig. 35.

Figure 35: Illustration of a vacancy and interstitial in a hexagonal lattice.

Because the interstitial sites in most crystalline solids are small (or have an unfavorable bonding configuration, as, for example, in the diamond lattice) interstitialcies are high-energy defects that are relatively uncommon.

Vacancies, on the other hand, are present in a significant concentration in all crystalline materials. Their most pronounced effect is to govern the migration of atoms on the crystal lattice (solid state diffusion). In order for an atom to move easily from one crystal lattice site to another the target site must be vacant.

Ordered compounds can have more complex intrinsic defects. In most compounds the different species are charged to at least some degree.

An intrinsic defect destroys the local charge balance, which must be restored in some way. The compound defects that preserve charge are easiest to visualize in binary ionic solids like NaCl. An isolated vacancy in an ionic solid creates an excess charge. The excess charge can be compensated by a paired vacancy on the sublattice of the other specie; for example, the excess charge associated with a Na vacancy is balanced if there is a Cl vacancy nearby.

A neutral defect that involves paired vacancies on the cation and anion sublattices is called a Schottky defect. The concentration of Schottky defects decreases the density of the crystal. Alternatively, the charge imbalance caused by the vacancy can be corrected by adding an interstitial of the same specie; a Na vacancy is compensated by a Na interstitial.

A neutral defect that is made up of a paired vacancy and interstitial is called a Frenkel defect. The concentration of Frenkel defects does not change the density of the crystal.

In compounds whose atoms are less strongly ionized it is energetically possible for species to exchange sites, so that an A-atom appears on the B sublattice or vice versa.

This type of point defect is called an anti-site defect, and is fairly common in semiconducting compounds such as GaAs.

Extrinsic defects

The extrinsic point defects are foreign atoms, which are called solutes if they are intentionally added to the material and are called impurities if they are not.

The foreign atom may occupy a lattice sites, in which case it is called a substitutional solute (or impurity) or it may fill an interstitial site, in which case it is called an interstitial solute. Since, the interstitial sites are relatively small, the type of the solute is largely determined by its size. Small atoms, such as hydrogen, carbon and nitrogen are often found in interstitial sites. Larger atoms are usually substitutional.

6.1.2 Line Defects - Dislocations

Dislocations are another type of defect in crystals. Dislocations are areas were the atoms are out of position in the crystal structure. Dislocations are generated and move when a stress is applied. It is responsible for the phenomenon of slip by which most metals deform plastically.

There are two basic types of dislocations, the edge dislocation and the screw dislocation (fig. 36).

Figure 36: Illustration of an edge and screw dislocation.

Edge Dislocation

In perfect crystal, atoms are arranged in both vertical and horizontal planes. If one of these vertical planes does not extend to full length, but ends in between within the crystal, then it is called edge dislocation.

Just above the edge of the complete plane the atoms are squeezed and are in a state of compression. Just below the edge of the incomplete plane, the atoms are pulled apart and are in the state of tension. The distorted configuration extends all along the edge into the crystal. The inter-atomic bonds are significantly distorted only in the immediate vicinity of the dislocation line. Edge dislocation are represented by Τ or ┴  depending on whether the incomplete plane starts from top or from the bottom of the crystal, called as positive or negative edge dislocation. The magnitude and the direction of the displacement are defined by a vector called Burgers vector. The Burgers vector is perpendicular to the edge dislocation line.

Burgers vector

To describe the size and the direction of the main lattice distortion caused by a dislocation a so-called Burgers vector b is introduced. To find the Burgers vector, a circuit from atom to atom counting the same number of atomic distances in all directions is made. If the circuit encloses a dislocation, it will not close. The vector that closes the loop is the Burgers vector (fig.37).

Figure 37: The Burgers circuit for an edge dislocation:

(A) Perfect crystal and

(B) crystal with distortion.

Screw Dislocations

There is a second basic type of dislocation, called screw dislocation. The motion of a screw dislocation is also a result of shear stress, but the defect line movement is perpendicular to direction of the stress and the atom displacement, rather than parallel. The Burgers vector is parallel to screw dislocation line. The speed of movement of a screw dislocation is lesser compared to edge dislocation.

6.1.3 Planar Defects

A disruption of the long-range stacking sequence can produce two other common types of crystal defects: 1) a stacking fault and 2) a twin boundary.

Stacking fault: A change in the stacking sequence over a few atomic spacing produces a stacking fault whereas a change over many atomic spacing produces a twin region. So, if the hcp structure is going along as ABABAB and suddenly switches to ABABABCABAB, there is a stacking fault present.

Twin boundary: Twin boundaries occur when two crystals of the same type intergrow, so that only a slight misorientation exists between them. It is a highly symmetrical interface, often with one crystal the mirror image of the other; also, atoms are shared by the two crystals at regular intervals. This is also a much lower-energy interface than the grain boundaries that form when crystals of arbitrary orientation grow together.

Twin boundaries occurs in pairs, such that the orientation change introduced by one boundary is restored by other. The region between the pair of boundaries is called twin region.

Another type of planer defect is the grain boundary.

Grains can range in size from nanometers to millimeters across and their orientations are usually rotated with respect to neighboring grains. Where one grain stops and another begins is known as a grain boundary. Grain boundaries limit the lengths and motions of dislocations. Therefore, having smaller grains (more grain boundary surface area) strengthens a material. The size of the grains can be controlled by the cooling rate when the material cast or heat treated. Generally, rapid cooling produces smaller grains whereas slow cooling result in larger grains.

6.1.4 Volume Defects

Volume defects occur on a much bigger scale than the rest of the crystal defects. Voids are regions where there are a large number of atoms missing from the lattice.

Another type of bulk defect occurs when impurity atoms cluster together to form small regions of a different phase. Other examples of bulk defects are pores, cracks and foreign particles.